# Tender Text: Erosion Control Mats for Slope Reinforcement

#### LV iGGmat straw coir erosion control mats with PP-net

Erosion control mats for the reinforcement of slopes, stitched erosion control mats made of 50 % straw and 50 % coir, double-sided PP net. Surface dimensions 350 - 450 g/m². Seeds: 25 g/m², RSM 7.1.1. Overlap width max 20 cm. Fastened with hardwood pegs, length 30 cm, amount 2-3 pieces/m². Tilt of the slope............1

Billing unit: m²

Deliver the erosion control mats and mount onto the prepared surface across the entire area and without any hollow spaces. The tracks are to be placed next to each other, vertically to the slope’s incline. The overlapping at the vertical longitudinal joints should amount to at least 10 cm, at least 10 – 20 cm for the cross joints. The open edge of the overlaps has to be averted from the main wind direction. Cross joints are generally to overlap from top to bottom or rather in the flowing direction of the water.

The top edge of the geotextile at the shoulder of the slope (............1 rm) is to be buried approx. 10 – 20 cm deep and fastened with 2 wooden pegs per meter of ditch.

The bottom edge of the geotextile at the foot of the slope (............1 rm) is to be buried approx. 10 – 20 cm deep and fixated with 2 wooden pegs per meter of ditch (billing for separate position no ............1).

Material:			stitched erosion control mat,

iGGmat SK-100-P2-PF-S-400 										double-sided PP-net
				with an inlay made of 50 % straw and 50 % coir

PP-stitch threads

seeds: 25 g/m², RSM 7.1.1

Weight per m²:		approx. 400 g (± 10 %)

Dimensions:			width 2.40 m, roll length 50.0 m

Max tensile strength, long:	2.8 kN/m

Max tensile strength, across:	1.4 kN/m

The fastening of the surface is to be implemented with 2–3 wood pegs per m².

Material:			wooden peg iGGfix beech Q30

Dimensions:			1 x 2 x 30 cm, with pre-mounted cross wood (cross wood: 5 cm)

Certificate:			Certificate for Sustainable Forestry

Proof of delivery:		Internationale Geotextil GmbH

Am Bahnhof 54

27239 Twistringen ∙ Germany

Tel. +49 (0) 4243 9288-20
				Fax +49 (0) 4243 9288-22

info@igg.de ∙ www.igg.de	                                                   or equivalent

............1 m²                 Unit price ............. €                  Total price............. €

1 = value is to be inserted by the client